M A T E R I A L   S A F E T Y   D A T A   S H E E T

TEXAS REFINERY CORP.

840 N. Main Street

Fort Worth, Texas  76164

(817) 332-1161

24 Hr. Emergency Telephone No.:   CHEMTREC 800-424-9300

Preparation/Revision Date – September, 2009

Supersedes - August, 2007

****** I.  PRODUCT IDENTIFICATION ******

TRADE NAME – SafetySolve II			    			PRODUCT CODE:  5800

CHEMICAL FAMILY - Chlorinated Solvent/Hydrocarbon Mixture

****** II.  HAZARDOUS INGREDIENTS ******

l.  Tetrachloroethylene *

CAS# 127-18-4          	PEL:  25 ppm          	TLV:  50 ppm

2.  Trichloroethylene

CAS# 79-01-6           	PEL:  100 ppm         	TLV:  50 ppm

3.  Carbon Dioxide (propellant)

CAS# 124-38-9          	PEL:  5000 ppm        	TLV:  5000 ppm

*  This product contains a toxic chemical(s) subject to the reporting requirements of section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR Part 372.  Each regulated chemical is present at a concentration that does not exceed the above specified upper bound concentration value.

****** III.  PHYSICAL/CHEMICAL DATA ******

BOILING POINT (OF/C):  165/74                 	  SPECIFIC GRAVITY (H20=2):  1.50

VAPOR PRESSURE (mm Hg):  59                     PERCENT VOLATILE BY VOL.:  100

VAPOR DENSITY (Air=1):  > 1                     SOLUBILITY IN WATER:  Negligible

EVAPORATION RATE (Butyl Acetate=1):  > 1

APPEARANCE AND ODOR – Light brown liquid; chlorinated solvent odor

****** IV.  FIRE AND EXPLOSION HAZARD DATA ******

FLASH POINT (OF/C):  None                              FLAMMABLE LIMITS:  N/A

EXTINGUISHING MEDIA:  Dry Chemical, CO2, Water Fog

SPECIAL FIRE FIGHTING PROCEDURES:  Use air-supplied breathing equipment.  Cool

exposed containers with water spray.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Aerosol cans at temperatures above 120oF

(49oC) may explode.  Treat as cylinders of compressed gas.

NFPA:         HEALTH:  2        FLAMMABILITY:  0        REACTIVITY:  1

HMIS:         HEALTH:  2        FLAMMABILITY:  0        REACTIVITY:  1    PPE: B

****** V.  HEALTH HAZARD DATA ******

TLV/PEL:  Not established - See Section II

CARCINOGENS - Perchloroethylene is classified as a Group C, possible human

carcinogen by IARC.  Perchloroethylene and trichloroethylene are known to cause

cancer by the State of California.

PRINCIPAL ROUTES OF EXPOSURE - Inhalation, Skin and Eyes

EFFECTS OF OVEREXPOSURE - Inhalation of high vapor concentrations is irritating to the respiratory system; may have results ranging from dizziness and headaches to unconsciousness.  May have other central nervous system effects.  Excessive exposure (chronic effects) may cause abnormalities in kidneys, spleen, lungs or brain.  May dry and de-fat the skin leading to irritation and dermatitis.  Will cause irritation to eyes.  Ingestion of product will cause nausea, vomiting, unconsciousness or even death.

EMERGENCY AND FIRST AID PROCEDURES - Remove to fresh air if inhaled.  Administer

oxygen if breathing is irregular or artificial respiration if breathing has stopped.  Remove contaminated clothing and wash skin with soap and water.  Flush eyes with water for 15 minutes; obtain medical attention if irritation persists.  If ingested, DO NOT induce vomiting.  Vomiting may lead to pneumonitis, which could be fatal.  Give two classes of water and get medical attention immediately.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE - Respiratory, cardiovascular or pulmonary illnesses

NOTES TO PHYSICIAN - None

****** VI.  REACTIVITY DATA ******

STABILITY - Stable

CONDITIONS TO AVOID - Avoid open flames, hot glowing surfaces or electric arcs.

INCOMPATABILITY (Materials to Avoid) - Strong oxidizers and strong acids.

HAZARDOUS DECOMPOSITION PRODUCTS - Thermal:  Oxides of carbon, phosgene, and

Hydrogen chloride gases.

HAZARDOUS POLYMERIZATION - Will not occur

****** VII.  PRECAUTIONS FOR SAFE HANDLING AND USE ******

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED - Add absorbent to spill area; pick up and place in closed container.

WASTE DISPOSAL METHOD - Conform to Federal, State and/or Local disposal regulations.  Do not puncture or incinerate the aerosol containers.  When all liquid is expelled, continue to depress button until gas has been exhausted.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE - Keep away from open flames, hot

surfaces or electric arcs.  Do not expose to temperatures above 120oF (49oC) due to contents being under pressure.

OTHER PRECAUTIONS - Vapor may collect in low lying areas

****** VIII.  CONTROL MEASURES ******

RESPIRATORY PROTECTION - None required with adequate ventilation.  Where vapor

concentrations exceed the TLV, respirator with organic canister should be used.

VENTILATION - Maintain concentrations below TLV of ingredients in Section II

PROTECTIVE GLOVES - Neoprene, viton or polyvinyl coated

EYE PROTECTION - Safety glasses with side shields or goggles

OTHER PROTECTIVE EQUIPMENT - None

WORK/HYGIENIC PRACTICES - Observe good personal hygiene practice when handling.

****** IX.  TRANSPORTATION DATA ******

DOT DESCRPTION - CONSUMER COMMODITY                      DOT HAZARD CLASS - ORM-D

FREIGHT CLASSIFICATION - NMFC 85980

FOR ADDITIONAL INFORMATION CONTACT:  Regulatory Affairs Office

(800)827-0711

THIS INFORMATION IS BEING SUPPLIED TO YOU UNDER OSHA "RIGHT TO KNOW" REGULATION

29 CFR 1910.1200 AND IS OFFERED IN GOOD FAITH.  THE INFORMATION CONTAINED HEREIN

IS BASED ON THE DATA AVAILABLE TO US AND IS BELIEVED TO BE TRUE AND ACCURATE.  NO

WARRANTY, EXPRESSED OR IMPLIED, REGARDING THE ACCURACY OF THIS DATA, THE HAZARDS

CONNECTED WITH THE USE OF THE MATERIAL, OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF, IS MADE.  TEXAS REFINERY CORP. ASSUMES NO RESPONSIBILITY FOR DAMAGE OR

INJURY FROM THE USE OF THE PRODUCT DESCRIBED HEREIN.