M A T E R I A L   S A F E T Y   D A T A   S H E E T

TEXAS REFINERY CORP.

840 N. Main Street

Fort Worth, Texas  76164

(817) 332-1161

24 Hr. Emergency Telephone No.:   CHEMTREC 800-424-9300

Preparation/Revision Date - September, 2007

Supersedes - September, 2006

****** I.  PRODUCT IDENTIFICATION ******

TRADE NAME - RED HOT

CHEMICAL FAMILY - Alkaline Cleaning Compound                PRODUCT CODE:  5300

****** II.  HAZARDOUS INGREDIENTS ******

Sodium Hydroxide

CAS# 1310-73-2               PEL/TLV:  2 mg/m                      WT.%  < 4

This material is not known to contain any chemicals on the SARA Section 313 list

at a concentration greater than 1.0% or any carcinogenic chemical at a concentration greater than 0.1%.

****** III.  PHYSICAL/CHEMICAL DATA ******

BOILING POINT (OF/C):  > 212/100                  SPECIFIC GRAVITY (H20=1):  1.11

VAPOR PRESSURE (mm Hg): as water                  VAPOR DENSITY (Air=1):  N/D

PERCENT VOLATILE BY VOL.:  88 - 89                SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (Butyl Acetate=1):  < 1          pH:  12.5 - 13.5

APPEARANCE AND ODOR - Clear, red liquid with slight detergent odor

****** IV.  FIRE AND EXPLOSION HAZARD DATA ******

FLASH POINT:  None                               FLAMMABLE LIMITS:  N/A

EXTINGUISHING MEDIA:  Dry chemicals or foam for fire adjacent to product storage

SPECIAL FIRE FIGHTING PROCEDURES:  Wear self-contained breathing apparatus for

chemical fires

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Contact with soft metals produces hydrogen

NFPA:       HEALTH:  2          FLAMMABILITY:  0          REACTIVITY:  0

HMIS:       HEALTH:  2          FLAMMABILITY:  0          REACTIVITY:  0

****** V.  HEALTH HAZARD DATA ******

TLV/PEL:  Not established - See Section II

PRINCIPAL ROUTES OF EXPOSURE - Skin and Eyes

CARCINOGENS - None known

EFFECTS OF OVEREXPOSURE - Prolonged contact may cause skin irritation or alkaline

burn.  Eye contact can cause burns and possible corneal damage.  Ingestion  may

result in irritation or damage to the digestive tract.  Prompt treatment is

important to minimize effects of exposure.  Inhalation may result in irritation or damage to the respiratory tract.

EMERGENCY AND FIRST AID PROCEDURES - Wash skin thoroughly with water.  Flush eyes

with water for 15 minutes; seek immediate medical attention.  If swallowed, give

one or two glasses of milk or water medical attention immediately.  Do not induce vomiting.  If inhaled, remove to fresh air.  Seek immediate medical attention.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE - Unknown

NOTES TO PHYSICIAN - None

****** VI.  REACTIVITY DATA ******

STABILITY - Stable

CONDITIONS TO AVOID - Contact with soft metals

INCOMPATABILITY (Materials to Avoid) - Strong acids and oxidizers

HAZARDOUS DECOMPOSITION PRODUCTS - Oxides of carbon from combustion

HAZARDOUS POLYMERIZATION - Will not occur

****** VII.  PRECAUTIONS FOR SAFE HANDLING AND USE ******

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED - Recover free liquid;

then apply absorbent material to remainder.  Place contaminated material in

approved waste container for disposal.

WASTE DISPOSAL METHOD - Conform to Federal, State and/or Local regulations for

disposal for cleaning compounds.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE - Keep from freezing.  Store at

temperatures between 40 - 120 F.  Keep in dry, cool, well-ventilated area.  Keep

container closed when not in use.

OTHER PRECAUTIONS - Keep out of reach of children.  Wash contaminated garments

****** VIII.  CONTROL MEASURES ******

RESPIRATORY PROTECTION - None normally required, but if misting could occur wear a NIOSH approved air purifying respirator.

VENTILATION - Local or mechanical

PROTECTIVE GLOVES - Rubber

EYE PROTECTION - Chemical goggles or shielded safety glasses

OTHER PROTECTIVE EQUIPMENT - Rubber boots and apron

WORK/HYGIENIC PRACTICES - Observe good personal hygiene practice when handling

this material.

****** IX.  TRANSPORTATION DATA ******

DOT SHIPPING NAME - CORROSIVE LIQUID, NOS (SODIUM HYDROXIDE)

DOT HAZARD CLASS - 8                                       DOT ID NUMBER - 1760

LABEL - CORROSIVE                                          PG III

FREIGHT CLASSIFICATION - Cleaning, Scouring and/or Washing Compound NOI

NMFC 48580 SUB 3 CLASS 55

* * * * * * * * * * * * *

FOR ADDITIONAL INFORMATION CONTACT:  Regulatory Affairs Office

(800) 827-0711   Ext. 277

* * * * * * * * * * * *

THIS INFORMATION IS BEING SUPPLIED TO YOU UNDER OSHA "RIGHT TO KNOW" REGULATION 29 CFR 1910.1200 AND IS OFFERED IN GOOD FAITH.  THE INFORMATION CONTAINED HEREIN IS BASED ON THE DATA AVAILABLE TO US AND IS BELIEVED TO BE TRUE AND ACCURATE.  NO

WARRANTY, EXPRESSED OR IMPLIED, REGARDING THE ACCURACY OF THIS DATA, THE HAZARDS

CONNECTED WITH THE USE OF THE MATERIAL, OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF, IS MADE.  TEXAS REFINERY CORP. ASSUMES NO RESPONSIBILITY FOR DAMAGE OR INJURY FROM THE USE OF THE PRODUCT DESCRIBED HEREIN.