Material Safety Data Sheet

Glendo Corporation

900 Overlander Road

P.O. Box 1153

Emporia, KS 66801-1153

Phone: 620-343-1084

Fax: 620-343-9640

I.	 Product Identification

Trade Name: 		Trim HM Part Numbers:

001-659, 002-167- 001-660

Date Prepared or Revised:	March 1, 2007

II.	Emergency Hazard Rating:

Health: 2

Fire: 2

Reactivity: 3

Specific Hazard: X

| CHEMICAL NAME | COMMON NAME | REG.* Y/N | CAS# | OSHA PEL | ACGIH TLV | CARCI-NOGEN** |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium Borate Hydrate | Borax | Y | 1303-96-4 | *** | 5mg/m3 | N |
| Sodium Nitrate | *** | Y | 07632-00-0 | *** | *** | N |

III. 	Physical and Chemical Data

| Boiling Point: N/A | Melting Point: N/A | Specific Gravity: N/A |
| --- | --- | --- |
| Vapor Pressure: N/A | Percent Volatile By Vol.: N/A | Vapor Density: N/A |
| Evaporation Rate: N/A | Solubility In Water: 5% | Solubility in Alcohol: N/A |
| Solubility in Other Solvent: N/A | Appearance and Odor: Off-white powder and mixture, slight odor |  |

IV.	Corrosivity and Reactivity Data

Stability: Stable

Polymerization: Will not Occur

Incompatability (Materials to Avoid): Reducing agents, flammable materials.

Decomposition Products: Nitrogen Oxides

Conditions to be avoided: Excessive heat, contact with flammable materials, electrical shock.

# Health, First Aid and Medical Data

| PRIMARY ROUTE(S) OF ENTRY: | EFFECTS OF OVEREXPOSURE: | FIRST AID AND MEDICAL INFORMATION: |
| --- | --- | --- |
| INHALATION: | May cause respiratory irritation | Remove to fresh air. Seek medical attention as needed |
| INGESTION: | May cause digestive irritation, nausea. | If conscious, induce vomiting. Seek medical attention as needed. |
| SKIN: | May cause irritation | Wash with mild soap and water. |
| EYE: | May cause irritation | Flush with water for 15 minutes. Seek medical attention immediately. |
| OTHER POTENTIAL HEALTH RISKS: | None Known. |  |

# Special Precautions in Handling/Storage

Precautions to be taken in Handling and Storage: Keep containers closed and

away from flammable materials. Store in a cool dry place.

Other Precautions: Utilize neoprene gloves, goggles and outer protective

covering.

VII.      Fire and Exposion Hazard Data

Flash Point: N/A Method Used: N/A Flammable Limits: N/A Lel: N/A UEL: N/A

Extinguishing Media: water spray or fog.

Special Fire Fighting Procedures: N/A

Explosion Potential: May react violently with reducers, flammables or electric shock.

VIII.     Spill, Leak and Disposal Procedures

Steps to be taken in case of Leaks or Spills: Collect dry spilled material. Use

water only for final clean-up.

Waste Disposal Method: Observe all applicable local, state and federal laws

and regulations.

- Regulatory Information

Proper Shipping Name: N/A

Shipping Label Requirement: N/A

The information contained herin is based on data considered accurate. However, no warranty is expressed or implied regarding the accuracy of these data or the results to be obtained from the use therof.

Vendor assumes no responsibility for injury to vendor or third persons proximately caused by the material if reasonable safety procedures are not adhered to as stipulated in the data sheet.

Additionally, vendor assumes no responsibility for injury to vendee or third persons proximately caused by abnormal use of the material even if reasonable safety procedures are followed. Furthermore, vendee assumes the risk in his use of the material.